SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Code of Conduct:

I _B.Samuel Jadon(192571042)_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

1)

Given Constraints

- CPU utilization < 15%

- Multiple sensors generate interrupts every 1 ms

- Vectored interrupt handling required

- Choose between memory-mapped and I/O-mapped I/O

Proposed Architecture

Sensors

S1 S2 S3 S4 ...

│

▼

Interrupt Controller

│

│ Vectored Interrupt

▼

CPU

│

┌─────┴─────┐

│           │

Memory       Sensor

Registers

Design

A vectored interrupt controller is connected between the sensors and CPU. Each sensor is assigned a unique interrupt vector. When a sensor generates an interrupt every 1 ms, the interrupt controller identifies the source and directly transfers control to the corresponding Interrupt Service Routine (ISR).

Memory-mapped I/O is selected because sensor registers can be accessed using normal load/store instructions. This provides simpler programming and low-latency access on modern embedded processors.

To maintain CPU utilization below 15%, the ISR should be very short. It should only read the sensor status/data, store it in a buffer, and clear the interrupt. Further processing is performed later using background tasks or DMA where applicable.

Operation

Sensor Interrupt

↓

Interrupt Controller

↓

Identify Interrupt Vector

↓

CPU saves context

↓

Execute corresponding ISR

↓

Read sensor data

↓

Clear interrupt

↓

Return to main program

Memory-mapped I/O + vectored interrupts provides low-latency communication while keeping CPU utilization below the required 15%.

2)

- Throughput > 3 GB/s

- NVMe over PCIe

- CPU should not handle bulk data transfer

- DMA with interrupt notification

Proposed Architecture

CPU

│

Command / Completion

│

▼

PCIe Root Complex

│

▼

NVMe Controller

│

DMA Engine

│

▼

Main Memory

│

▼

NVMe SSD

Design

An NVMe SSD connected through PCIe is used to achieve throughput greater than 3 GB/s. The NVMe controller uses DMA (Direct Memory Access) to transfer data directly between the SSD and main memory.

The CPU only prepares NVMe commands and manages the command queues. It does not copy the actual bulk data.

After completing a DMA transfer, the NVMe controller generates an interrupt to notify the CPU.

Data Transfer

CPU creates NVMe command

↓

NVMe Controller

↓

DMA Transfer

↓

Main Memory

↓

Interrupt

↓

CPU processes completion

Advantages

- Very high data-transfer rate

- Reduced CPU overhead

- Efficient use of PCIe bandwidth

- DMA handles bulk transfers

- Interrupts notify CPU only after important operations

NVMe + PCIe + DMA + interrupt notification is suitable for achieving more than 3 GB/s while minimizing CPU involvement.

3)

Given Constraints

- 6 USB peripherals

- 2 high-speed Thunderbolt devices

- Avoid interrupt starvation

- Hardware and software interrupts

- Fair bus arbitration

Proposed Architecture

CPU

│

Interrupt Controller

/             \

/               \

USB Controller    Thunderbolt

│              Controller

┌───┼───┐           ┌───┴───┐

USB USB USB          TB      TB

│   │   │

USB USB USB

Design

A USB host controller manages the six USB peripherals, while a Thunderbolt controller manages the two high-speed devices.

A centralized interrupt controller prioritizes hardware interrupts. Critical device interrupts are assigned higher priority, while less important interrupts are handled at lower priority.

Software interrupts are used for operating-system services and deferred processing.

A fair bus arbitration mechanism, such as round-robin arbitration, is used so that no peripheral continuously occupies the bus.

Interrupt Handling

High-priority hardware interrupt

↓

Interrupt Controller

↓

CPU → ISR

Software interrupt

↓

OS service routine

Avoiding Interrupt Starvation

- Use interrupt priorities.

- Limit ISR execution time.

- Use interrupt coalescing where possible.

- Use round-robin bus arbitration.

- Defer lengthy processing to software tasks.

A combination of USB/Thunderbolt controllers, prioritized interrupts, software interrupts, and round-robin arbitration ensures efficient and fair communication without interrupt starvation.

4)

Given Constraints

- Continuous data stream = 500 MB/s

- Minimal CPU intervention

- Compare DMA and interrupt-driven I/O

- Memory-mapped device registers

Proposed Architecture

Data Acquisition Device

│

Memory-Mapped Registers

│

▼

DMA Engine

│

▼

Main Memory

│

▼

CPU

DMA vs Interrupt-Driven I/O

| Feature | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | High |
| Suitable for 500 MB/s | Yes | No |
| Data transfer | Directly to memory | CPU handles transfer |
| CPU overhead | Low | High |
| Throughput | High | Lower |
| Efficiency | Excellent | Poor for continuous streams |

Design

DMA is selected because the system continuously receives 500 MB/s. The device places data into memory through DMA without requiring the CPU to transfer every data word.

Device control and status registers are accessed using memory-mapped I/O.

The DMA controller generates an interrupt after transferring a block of data rather than generating an interrupt for every byte or word.

Operation

Device

↓

DMA Controller

↓

Memory Buffer

↓

Interrupt CPU after block transfer

↓

CPU processes completed buffer

DMA is significantly better than interrupt-driven I/O for a continuous 500 MB/s data stream because it minimizes CPU intervention and provides high throughput.

5)

Given Constraints

- Nested interrupts

- Maximum latency < 10 µs

- Vectored interrupts for high-priority devices

- Software interrupts for background tasks

Proposed Architecture

Devices

│

┌───────┴────────┐

│ Interrupt      │

│ Controller     │

└───────┬────────┘

│

Priority Encoder

│

▼

CPU

┌─────┴─────┐

│           │

High Priority   Low Priority

Hardware      Hardware

Interrupt     Interrupt

│

▼

ISR 1

│

Higher Priority

Interrupt

↓

ISR 2

↓

Return to ISR 1

↓

Return to program

Design

A priority-based interrupt controller is used to support nested interrupts. High-priority devices are assigned unique interrupt vectors.

When a high-priority interrupt occurs, the CPU saves the current context and executes its ISR. If an even higher-priority interrupt occurs while the ISR is running, the current ISR can be temporarily suspended and the higher-priority ISR is executed.

Software interrupts are reserved for background operating-system services and non-critical tasks.

Meeting the 10 µs Requirement

To keep interrupt latency below 10 µs:

- Use vectored interrupts.

- Keep ISRs short.

- Use hardware priority arbitration.

- Avoid disabling interrupts for long periods.

- Use DMA for large data transfers.

- Defer lengthy processing to background tasks.

Interrupt Priority

Priority 1 → Critical real-time device

Priority 2 → High-speed I/O

Priority 3 → Normal device

Priority 4 → Background software interrupt

A priority-based vectored interrupt controller with nested interrupts provides fast response to critical devices. Keeping ISRs short and using DMA for bulk transfers helps maintain the required < 10 µs maximum interrupt latency.